The dual core sample code consists of two projects:

One for the A5 (dual_core_tower), and one for the M4 (dual_core_m4_tower).

The M4 code gets compiled into a binary with an executable code start address at 0x3f040410. The A5 project automatically loads that binary as part of the linker setup, and then points the M4 core to that address. The linker files are built to split the SystemRAM in half between the two cores.

Open the \build\iar\projects\dual_core workspace to see both the A5 and M4 projects.

The key part of the M4 project is the linker file, so that the start address is always the same (0x3f040410), and the code will be loaded at 0x3f040000 every time.

The key pieces of code for the A5 project are:

SRC->GPR2 = (unsigned int)0x3f040411; //Set M4 start address. Must be odd address since M4 is

thumb

CCM->CCOWR = 0x15a5a;                          //Enable M4 core

Then in the A5 linker file, there’s two key lines:

define symbol __code_start_m4__ = 0x3f040000;

place at address mem:__code_start_m4__ { readonly section .noinit_m4 };

This tells the linker where to place the M4 binary code.

Finally the linker input configuration in the A5 project tells the linker to include the M4 binary code as part of the ELF it generates.